EOL MES + Camera CMS

User Manual

Toyota Boshoku Device India  •  Bawal Plant  •  YNC SS Line

Maintainer: Vivek Kumar    |    Last updated: 2026-05-07

1. Purpose & Methodology

What was needed

The YNC seat-slider line had to track production, downtime, quality and process metrics in real time, so operators on the floor and managers in the office both see the same numbers without anyone manually re-entering data. We also needed a camera system that records every cycle so any defect can be replayed.

Approach used

One PLC (Mitsubishi Q-series, MC4E protocol on TCP) is the ground-truth source. It sets bits like L108 = OK, L109 = NG and word registers like D6005 = status code, D6048 = current model number.

A small Python service called the Collector polls the PLC every 30 ms, debounces the bits, and writes finished cycles into PostgreSQL. The collector is the only program that talks to the PLC — everything else just reads the DB.

A FastAPI backend turns DB rows into clean JSON for the browser.

A React + Vite frontend is the actual screen — operator dashboard, admin panel, and fullscreen TV view on the shop floor.

The Camera CMS is a parallel mini-app: it owns the cameras, records continuous .ts video per camera, and on demand cuts a clip of any cycle a user clicks on. It pulls Zone/Line/Machine names from the MES so both systems show the same hierarchy.

The whole stack is on one PC for now (192.168.10.185). Everything is reachable over LAN, so any PC on the factory network can open the dashboard in a browser.

2. End-to-End Workflow (PLC → Screen)

The data flow from the physical PLC bit going high to the operator seeing it on screen takes ~1-2 seconds end to end. The chain:

PLC                  Collector                PostgreSQL
  ─────                ──────────               ──────────
  L108 / L109     ─►   reads bit          ─►   ync_dashboard_complete
  D6005 status    ─►   reads word         ─►   ync_dashboard_complete_ct_log
  D6048 model     ─►   reads word         ─►   ync_status_log
  M-bit (sub)     ─►   edge count         ─►   ync_seatslider
                                                  │
                                                  ▼
                                         FastAPI (port 8080)
                                            GET /api/lines/2/realtime
                                            GET /api/lines/2/ct-history
                                                  │
                                                  ▼
                                         React Frontend (port 5656)
                                            Dashboard / Admin / Fullscreen

For video, the chain is parallel:

Hikvision RTSP camera ─► ffmpeg (24×7) ─► F:\CameraCMS_Videos\<cam>\<shift>.ts
                                                       │
                                                       ▼ (on user click)
                                          ffmpeg -ss/-to clip ─► MP4 player

3. Directory Structure

Root project: D:\EOL\EOL\Deep (2)\Deep\

| Path | Purpose |
| --- | --- |
| Phase2\ | MES backend (FastAPI). The brains. |
| Phase2\main.py | App entrypoint, DB migrations, CMS proxy endpoints. |
| Phase2\auth.py | JWT login + role checks. |
| Phase2\database.py | Single Postgres connection helper. |
| Phase2\collector_engine.py | PLC poller class — only file that reads MC4E bits. |
| Phase2\collectors\collector_ync_l6.py | Bootstrap script that launches engine for Line 2. |
| Phase2\routers\*.py | One file per page area: lines, breakdowns, users, quality, etc. |
| mes-frontend\ | MES React frontend (Vite). |
| mes-frontend\src\pages\Dashboard.jsx | Live shop-floor dashboard. |
| mes-frontend\src\pages\Fullscreen.jsx | TV display (no login required). |
| mes-frontend\src\pages\AdminPanel.jsx | All admin pages: users, lines, machines, cameras. |
| mes-frontend\src\pages\Historical.jsx | Past-shift analysis. |
| mes-frontend\src\pages\ProcessGraphs.jsx | Sub-machine cycle pulse graphs. |
| mes-frontend\src\context\AuthContext.jsx | Per-tab session + JWT. |
| mes-frontend\src\api\client.jsx | Axios wrapper used by every page. |
| Camera CMS\ | Camera CMS (Flask + React, separate stack). |
| Camera CMS\backend\api_server.py | Flask API on :5000 + RecordingManager auto-start. |
| Camera CMS\backend\recorder_engine.py | ffmpeg-based per-camera continuous .ts recorder. |
| Camera CMS\backend\cycle_events.py | Append-only cycles.csv log. |
| Camera CMS\backend\plc_poller.py | Scaffold — reads L108/L109/M-bit at 30 Hz. |
| Camera CMS\backend\settings_config.py | Stores user-chosen video storage path. |
| Camera CMS\backend\mes_sync.py | One-way pull of zones/lines/machines from MES. |
| Camera CMS\backend\bin\ffmpeg.exe | Bundled ffmpeg used for record + clip extract. |
| Camera CMS\backend\zones.json | CMS Zone/Line/Machine tree (synced from MES). |
| Camera CMS\backend\users.json | Local CMS users. Default admin / admin123. |
| Camera CMS\backend\cycles.csv | Append-only cycle log. |
| Camera CMS\frontend\src\pages\masters\CameraMaster.jsx | Camera CRUD with Mounted-On column. |
| Camera CMS\frontend\src\pages\config\CameraConfig.jsx | Bind machine ↔ camera ↔ PLC. |
| Camera CMS\frontend\src\pages\config\SystemSettings.jsx | Storage path + Sync from MES button. |
| start_everything.bat | THE one launcher. Starts MES + collector + CMS + frontends. |
| stop_everything.bat | Kills every Python / Node / ffmpeg process. |

4. Network Configuration (IP-by-IP)

Server PC: 192.168.10.185 (also reachable as 127.0.0.1).

| IP | Port | Service | Notes |
| --- | --- | --- | --- |
| 192.168.10.185 | 8080 | MES Backend (FastAPI) | All MES API endpoints |
| 192.168.10.185 | 5656 | MES Frontend (Vite) | Operator dashboard, admin |
| 192.168.10.185 | 5000 | Camera CMS API (Flask) | Camera + cycle endpoints |
| 192.168.10.185 | 8050 | Camera CMS MJPEG Streams | Live preview tiles |
| 192.168.10.185 | 5173 | Camera CMS Frontend (Vite) | CMS admin portal |
| 192.168.10.210 | 5432 | PostgreSQL (energydb) | All persistent data. user postgres / tbdi@123 |
| 192.168.10.150 | 5002 | Main PLC (Mitsubishi MC4E) | YNC-SS line. L108 = OK, L109 = NG |
| 192.168.10.190 | 5002 | Sub-PLC #1 — Upper Rail Greasing M/c | M100 cycle pulse |
| 192.168.10.191 | 5002 | Sub-PLC #2 — Lock Bar Insert M/c | X3 cycle pulse |
| 192.168.10.192 | 5002 | Sub-PLC #3 — Lower Rail Greasing M/c | M100 cycle pulse |
| 192.168.10.115 | 554 | Hikvision RTSP camera (final station) | User admin / admin123 |

Storage: cycle videos go to F:\CameraCMS_Videos\ on the external 4 TB HDD. Path is configurable from CMS → Configuration → System Settings.

5. Page → Data → Database Mapping

Each MES page calls a few API endpoints, which read specific tables. The most important mappings:

| MES Page | API Endpoint | DB Table(s) |
| --- | --- | --- |
| Dashboard (live OEE, plan vs actual) | GET /api/lines/2/realtime | ync_dashboard_complete, mes_lines |
| Cycle-Time history graph | GET /api/lines/2/ct-history | ync_dashboard_complete_ct_log |
| Loss Distribution panel | GET /api/lines/2/hourly-loss-breakdown | ync_status_log |
| Shift timeline strip | GET /api/lines/2/status-log | ync_status_log |
| Fullscreen TV (no login) | (same endpoints, anonymous) | (same tables) |
| Quality Dashboard / Poka-Yoke | GET /api/poka-yoke/live/2 | mes_poka_yoke_events, mes_py_master |
| Maintenance Dashboard | GET /api/breakdowns/pending-production | mes_breakdowns |
| Process Graphs (sub-machines) | GET /api/submachines/{id}/hourly | mes_machine_process_log |
| Production Hourly History | GET /api/lines/2/production_history | ync_hourly_production |
| Admin Panel — Cameras list | GET /api/cms/cameras (proxy) | via CMS cameras.json |
| AI Assistant chat | POST /api/ai/chat | reads any of the above |

Camera CMS pages and their stores:

| CMS Page | Stored In |
| --- | --- |
| Camera Master | cameras.json (Fernet-encrypted creds) |
| PLC Master | plcs.json |
| Zone / Line / Machine Master | zones.json (synced from MES Postgres) |
| Camera Config (bindings) | camera_config_bindings.json |
| Shift Config | shifts.json |
| Cycle Monitor / Reports | cycles.csv (one row per cycle event) |
| System Settings | settings.json |

6. How to Start the System

One file does it all.

Plug in the F:\ external HDD.

Double-click  D:\EOL\EOL\Deep (2)\Deep\start_everything.bat

Six command-prompt windows open in this order:

1. MES-API — uvicorn on :8080

2. MES-Collector — PLC poll loop, prints [STATUS] IDLE → RUNNING etc.

3. MES-Frontend — Vite on :5656

4. CMS-API — Flask on :5000 (also brings up RecordingManager)

5. CMS-Streams — MJPEG on :8050

6. CMS-Frontend — Vite on :5173

Browser auto-opens to http://127.0.0.1:5656/

Login with: admin / admin123

To stop everything cleanly: double-click stop_everything.bat

If a single service hangs, just close its window; the others keep running. The collector singleton lock auto-releases after 30 s of no heartbeat, so a crashed collector can be restarted safely without manual cleanup.

LAN-visible URLs:

MES dashboard: http://192.168.10.185:5656

CMS portal:    http://192.168.10.185:5173

7. User Roles & Access

There are six roles. Every login JWT carries the role; the backend enforces require_role(...) per endpoint and the frontend hides / shows pages based on the same check, so an operator never even sees an admin button.

Role colour scheme (top-banner accent on dashboard)

| Role | Colour | Used on |
| --- | --- | --- |
| Admin / Plant Head | Blue | Admin Panel chrome, ADMIN badge |
| Production | Green | Production dashboard, hourly grid header |
| Maintenance | Red | Breakdown banner, MTTR/MTBF tiles |
| Quality | Amber / Yellow | Poka-yoke live, NG counter pills |
| Operator | Grey | Default tile, light-mode shop floor |
| Department (generic) | Slate | Department-portal landing |

The dashboard layout uses these colours as the four corner accents (TL = Production green, TR = Quality amber, BL = Maintenance red, BR = Admin blue) so anyone walking past the screen can tell at a glance which department's number is healthy.

Page access matrix

| Page | Adm | PH | Prod | Maint | Qual | Op | Dept |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Login | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |
| Dashboard | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |
| Fullscreen TV (no login) | — | — | — | — | — | — | — |
| Cycle-Time History | ✓ | ✓ | ✓ | ✓ | ✓ | RO | ✓ |
| Loss Distribution | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |
| Hourly Production Grid | ✓ | ✓ | ✓ | ✓ | ✓ | RO | ✓ |
| Process Graphs | ✓ | ✓ | ✓ | ✓ | ✓ | RO | ✓ |
| Quality Dashboard / Poka-Yoke | ✓ | ✓ | RO | RO | ✓ | RO | ✓ |
| Maintenance Dashboard | ✓ | ✓ | RO | ✓ | RO | RO | ✓ |
| Breakdown raise / fill | ✓ | ✓ | P | M | ✗ | ✗ | ✗ |
| CAPA / Deviations | ✓ | ✓ | ✗ | M | Q | ✗ | own |
| Admin → Users | ✓ | ✓ | ✗ | ✗ | ✗ | ✗ | ✗ |
| Admin → Lines / Plants | ✓ | ✓ | ✗ | ✗ | ✗ | ✗ | ✗ |
| Admin → Cameras list | ✓ | ✓ | ✗ | ✗ | ✗ | ✗ | ✗ |
| Admin → Slip Threshold | ✓ | ✓ | ✗ | ✗ | ✗ | ✗ | ✗ |
| AI Assistant chat | ✓ | ✓ | ✓ | ✓ | ✓ | ✗ | ✓ |
| Camera CMS portal | ✓ | ✓ | ✗ | ✗ | ✗ | ✗ | ✗ |

Legend:  ✓ full   |   RO read-only   |   P production-half-fill   |   M maintenance-half   |   Q quality-only   |   own = department's own only

Admin can also issue per-user, per-page overrides via Admin Panel → Users → Permissions Matrix (none / read / full columns × every page). Those overrides win over the default role mapping above.

Default seeded credentials

| User | Password | Role |
| --- | --- | --- |
| admin | admin123 | admin |
| supervisor | super123 | supervisor / plant_head |
| operator | oper123 | operator |

Change these immediately in production via Admin Panel → Users → Change Password.

End of manual.   For internal questions contact Vivek Kumar (Manufacturing & Automation Engineer, TBDI Bawal).